WP 1.2  Quadrupoles for TL1+TL2 from LURE / 			ORSAY

SUPER-ACO     	 800 MeV   e+  STORAGE RING FOR SYNCHROTRON 				 RADIATION LIGHT SOURCE

-  started 1987			-  stopped end 2003

-  available quadrupoles:	32

-  type:				solid non-laminated yokes for DC operation

-  max. gradient:		8.4 T/m  @ 475 A

-  water cooled coils with 29 windings

-  length :			320 mm

-  inscribed diameter:		136 mm

-  accessibility for dismantling:

24 units more easily accessible

8 units more difficult (additional demounting of shielding 					     or injection beam line etc)

What is needed to get them to CERN:

- official request from R. Aimar to A. Tadjeddine/LURE

- MoU on conditions to cease property

- INB procedure

- CERN in charge of  all dismantling costs + transport to CERN

- CERN in charge for payment for assistance by LURE personnel